CHAPTER 2

REVIEW OF RELATED LITERATURE

Smart Water Systems: Reviews and Surveys

Okoli and Kabaso (2024) explain that smart water systems help cities improve water distribution, detect leaks, and monitor usage in real time. Their review found that current smart water prototypes commonly use microcontrollers, embedded programming, sensors, and wireless modules, but none have yet utilized 5G connectivity showing an opportunity for future systems. They also highlighted the emerging use of 3D printing and solar power as sustainable enhancements.

Farah and Shahrour (2024) provided a comprehensive analysis of water leak detection technologies across 23 years of research. They classified methods from traditional (manual sounding) to advanced smart sensor networks. Their bibliometric review emphasized that leak detection is a global concern due to the economic, social, and environmental damages caused by undetected leakage. However, gaps remain in accuracy, real-time monitoring, and cost-effectiveness.

Similarly, Yussof and Ho (2022) examined the state-of-the-art leak detection methods for smart buildings. Their review emphasized that while significant progress has been made, real-time models and automated detection systems in building automation are still limited. The authors concluded the need for improved integration of leakage detection, evaluation, and control systems to minimize human intervention.

IoT Metering, LPWAN, and Connectivity Technologies

The advancement of IoT has introduced low-power communication networks suited for large-scale water monitoring. LoRaWAN, a leading Low-Power Wide-Area Network (LPWAN), is widely explored because it offers long-range communication at low energy cost.

Alghamdi et al. (2022) analyzed the performance of LoRaWAN in a housing complex for water leakage detection. Their simulation-based research demonstrated that LoRaWAN performs efficiently when nodes are strategically distributed, showing high feasibility for large residential applications.

Lei et al. (2025) designed a LoRa-enabled smart water meter using an STM32 microcontroller, flow sensors, and servo-driven water valve control. Their work demonstrated highly stable and accurate transmission of usage data—reducing manual meter reading and improving operational efficiency.

Pagano et al. (2024) introduced SWI-FEED, a large-scale IoT framework using EPANET and NS-3 simulation tools to optimize water distribution networks. The study focused on gateway placement, energy efficiency, and massive IoT deployments, proving that large-scale smart water systems could be optimized through hybrid simulation tools.

Leak Detection Using Machine Learning, TinyML, and Edge Analytics

Machine Learning (ML) and TinyML are becoming crucial components of modern water management systems, especially for leak detection in real time.

Atanane et al. (2023) proposed a TinyML-based water leakage detection system for smart buildings. Their approach used acoustic leak datasets and Convolutional Neural Networks (CNN)—including EfficientNet, ResNet, AlexNet, and MobileNet variants. EfficientNet achieved over 97% accuracy after model optimization. The final compressed model was deployed on an Arduino Nano 33 BLE, demonstrating that leak detection can run effectively on low-power edge devices.

Commercial case examples also show the growing adoption of smart metering. The 2022 LoRa Alliance case study highlighted the need for commercial-grade water metering solutions due to insurance and building management requirements. LoRaWAN-enabled water meters provide continuous consumption monitoring and early leak alarms, making buildings more resilient and cost-efficient.

Cloud Ingestion, Dashboards, SCADA, and Backend Architectures

Cloud computing has enabled real-time data acquisition and visualization for water quality and consumption monitoring.

Bukhair et al. (2023) developed a cloud-based water quality monitoring system using ESP32, sensors, and Google Firebase. Their system allows users to view real-time parameters (pH, turbidity, conductivity) via a mobile app, overcoming the limitations of laboratory-based testing. The authors emphasized the applicability of the system in environmental protection, public health, and water resource conservation.

Recent studies (Farah & Shahrour, 2024) on leak detection also highlighted the importance of centralized and cloud-integrated dashboards that gather sensor data from pipelines to support accurate burst detection and monitoring.

Singh and Walingo (2024) proposed an IoT WSN structure that uses AI-based virtual sensors to estimate water quality indicators that lack physical sensors. Their machine learning models predicted E. coli concentrations using other water parameters, demonstrating the potential of AI to expand the measurement capabilities of water monitoring systems.

Electronic Billing, E-Payments, and Digital Utility Systems

Digital transformation in utilities has expanded to billing and customer management systems.

Mulmane and Korbu (2023) proposed an IoT-enabled digital water billing system using NodeMCU, water flow sensors, and ThingSpeak cloud services. Their design automates monthly readings, eliminates human error, and provides real-time consumption visibility for customers and water providers.

Siddique and Shaik (2024) examined the meter-to-cash lifecycle for utilities using SAP S/4HANA. Their study detailed how digital billing systems streamline metering, payment processing, and revenue collection. The paper highlights how advanced ERP tools can modernize utility operations and improve customer satisfaction.

Amankwaa et al. (2024) explored digital transformation in Ghana’s public water utility, showing that digitalization enhances operational efficiency but may introduce internal power shifts and stakeholder challenges. Their socio-technical analysis revealed that impacts are incremental rather than transformative, suggesting the need for long-term digital maturity strategies.

Applied Case Studies in Urban and Smart City Environments

Several case studies highlight real-world deployments of smart water systems.

Andrić et al. (2022) demonstrated a smart building system capable of real-time monitoring of water consumption using IoT sensors and analytics. The system helps users visualize consumption patterns, reduce wastage, and promote sustainable water usage—showing how IoT can influence user behavior.

Universiti Teknologi Malaysia (Musa et al., 2022) implemented a smart water monitoring prototype using IoT and GIS integration. Their system included four layers: physical, network, processing, and application. Results showed real-time relationships between water pressure and flow rate, illustrating the effectiveness of cloud-based IoT monitoring in university campuses or city districts.

These case studies reinforce the potential of IoT for large-scale smart city applications in water conservation.

Standards, Performance, and Technical Evaluation

Performance evaluations and standards guide the proper deployment of IoT water systems.

Among the key technical evaluations is the work by Syed Taha et al. (2024), which assessed LoRa performance for water quality monitoring. Their results showed that factor selection (spreading factor, distance, gateway deployment) significantly affects transmission reliability—critical for continuous sensing applications.

Similarly, Alghamdi et al. (2022) emphasized that network scalability and signal reliability must be considered when deploying LoRaWAN in residential networks.

Standards-based frameworks and performance evaluations guide engineers in proper system sizing, sensor selection, network design, and cloud architecture—ensuring smart water systems remain reliable in real-world conditions.

REFERENCES
	Okoli, N. J., & Kabaso, B. (2024). Building a smart water city: IoT smart water technologies, applications, and future directions. Water, 16(4), 557. https://tinyurl.com/bdhsfv6w

Farah, E., & Shahrour, I. (2024). Water leak detection: A comprehensive review of methods, challenges, and future directions. Water, 16(20), 2975. https://tinyurl.com/5enu9626

Yussof, N. A. M., & Ho, H. W. (2022). Review of water leak detection methods in smart building applications. Buildings, 12(10), 1535. https://tinyurl.com/37z7euza
	Alghamdi, A. M., Khairullah, E. F., & Al mojamed, M. M. (2022). LoRaWAN performance analysis for a water monitoring and leakage detection system in a housing complex. Sensors, 22(19), 7188. https://tinyurl.com/3yhvm6tw

Lei, Z., Xie, Y., & Ji, X. (2025). Design and implementation of smart IoT water meter based on LoRa. In EITCE '24: Proceedings of the 2024 8th International Conference on Electronic Information Technology and Computer Engineering (pp. 1447–1451). ACM. https://doi.org/10.1145/3711129.3711371

Pagano, A., Garlisi, D., Giuliano, F., Cattai, T., & Cuomo, F. (2024). SWI-FEED: Smart water IoT framework for evaluation of energy and data in massive scenarios. arXiv. https://tinyurl.com/5n8rtckd

Atanane, O., Mourhir, A., Benamar, N., & Zennaro, M. (2023). Smart buildings: Water leakage detection using TinyML. Sensors, 23(22), 9210. https://tinyurl.com/57rupf7x

LoRa Alliance. (2022). Smart building water metering with LoRaWAN: 2022 use case. https://tinyurl.com/nhdntycu

Bukhair, M. S., Tufail, S. M., Sultan, S., & Ansari, M. Z. (2023). Development of cloud-based water quality monitoring system. ResearchGate. https://www.researchgate.net/publication/373635179_Development_of_Cloud-based_Water_Quality_Monitoring_System

Farah, E., & Shahrour, I. (2024). Water leak detection: A comprehensive review of methods, challenges, and future directions. ResearchGate. https://www.researchgate.net/publication/385054815_Water_Leak_Detection_A_Comprehensive_Review_of_Methods_Challenges_and_Future_Directions

Singh, Y., & Walingo, T. (2024). Smart water quality monitoring with IoT wireless sensor networks. Sensors, 24(9), 2871. https://tinyurl.com/4v7wp9x5

Mulmane, S. R., & Korbu, S. (2023). Cloud-based smart digital water billing meter.

World Bank. (2023). Digital progress and trends report 2023. https://tinyurl.com/2n96zfc6

World Bank. (n.d.). Digital water: Supporting water and supply and sanitation utilities in their digital journey. https://tinyurl.com/4j5hvcwj

Siddque, K. Q., & Shaik, M. (2024). Digital transformation in utilities: Exploring SAP solutions for meter-to-cash processes in utilities. Journal of Computer Sciences and Applications, 12(1), 1–9. https://tinyurl.com/6hhshnbk

Amankwaa, G., Heeks, R., & Browne, A. L. (2024). Powershifts, organisational value, and water management: Digital transformation of Ghana's public water utility. Journal of 	Utilities Policy, 87, 101724. https://doi.org/10.1016/j.jup.2024.101724

Andrić, I., Vrsalović, A., Perković, T., Aglić Čuvić, M., & Šolić, P. (2022). IoT approach towards smart water usage. Journal of Cleaner Production, 365, 132792. https://doi.org/10.1016/j.jclepro.2022.132792

Abdul Aziz, N. A. A., Musa, T. A., Musliman, I. A., Omar, A. H., & Wan Aris, W. A. (2022). Smart water network monitoring: A case study at Universiti Teknologi Malaysia. [Conference paper].

Alghamdi, A. M., Khairullah, E. F., & Almojamed, M. M. (2022). LoRaWAN performance analysis for a water monitoring and leakage detection system in a housing complex. Sensors, 22(19), 7188. https://doi.org/10.3390/s22197188

Syed Taha, S. N., Abu Talip, M. S., Mohamad, M., Azizul Hasan, Z. H., & Tengku Mohmed Noor Izam, T. F. T. (2024). Evaluation of LoRa network performance for water quality monitoring systems. Applied Sciences, 14(16), 7136. https://doi.org/10.3390/app14167136

Rajab, H., Al-Amaireh, H., Bouguera, T., & Cinkler, T. (2023). Evaluation of energy consumption of LPWAN technologies. EURASIP Journal on Wireless Communications and Networking, 2023, Article 118. https://doi.org/10.1186/s13638-023-02322-8